CODE:

//=====================================================================

// Description: SystemVerilog Assertions for the synchronous FIFO.

//              Bound to the fifo module so that internal signals

//              (wr_ptr, rd_ptr) are directly visible without

//              modifying the RTL port list.

//=====================================================================

module fifo_assertions #(

parameter int DATA_WIDTH = 8,

parameter int DEPTH      = 8,

parameter int PTR_WIDTH  = $clog2(DEPTH)

)(

input logic                   clk,

input logic                   rst_n,

input logic                   wr_en,

input logic                   rd_en,

input logic [DATA_WIDTH-1:0]  din,

input logic [DATA_WIDTH-1:0]  dout,

input logic                   full,

input logic                   empty,

input logic [PTR_WIDTH:0]     wr_ptr,

input logic [PTR_WIDTH:0]     rd_ptr

);

//=================================================================

// Reference model (golden queue) for data-ordering checks

//=================================================================

logic [DATA_WIDTH-1:0] ref_q[$];

logic [DATA_WIDTH-1:0] expected_data;

logic                  expected_valid;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

ref_q.delete();

expected_data  <= '0;

expected_valid <= 1'b0;

end else begin

// Model a valid write

if (wr_en && !full) ref_q.push_back(din);

// Model a valid read (dout is registered, valid 1 cycle later)

if (rd_en && !empty) begin

expected_data  <= ref_q.pop_front();

expected_valid <= 1'b1;

end else begin

expected_valid <= 1'b0;

end

end

end

//=================================================================

// Property 1: FIFO shall be empty after reset

//=================================================================

property p_reset_empty;

@(posedge clk) $rose(rst_n) |-> empty;

endproperty

a_reset_empty: assert property (p_reset_empty)

else $error("[ASSERT FAIL] FIFO not empty immediately after reset");

//=================================================================

// Property 2: full and empty shall not be asserted simultaneously

//=================================================================

property p_full_empty_mutex;

@(posedge clk) disable iff (!rst_n) !(full && empty);

endproperty

a_full_empty_mutex: assert property (p_full_empty_mutex)

else $error("[ASSERT FAIL] full and empty asserted at the same time");

//=================================================================

// Property 3: A read shall not be performed when FIFO is empty

// (i.e., an rd_en while empty must have NO effect on rd_ptr)

//=================================================================

property p_illegal_read_ignored;

@(posedge clk) disable iff (!rst_n)

(rd_en && empty) |=> (rd_ptr == $past(rd_ptr));

endproperty

a_illegal_read_ignored: assert property (p_illegal_read_ignored)

else $error("[ASSERT FAIL] rd_ptr changed during illegal read (empty)");

cov_illegal_read: cover property (

@(posedge clk) disable iff (!rst_n) (rd_en && empty));

//=================================================================

// Property 4: A write shall not be performed when FIFO is full

// (i.e., a wr_en while full must have NO effect on wr_ptr)

//=================================================================

property p_illegal_write_ignored;

@(posedge clk) disable iff (!rst_n)

(wr_en && full) |=> (wr_ptr == $past(wr_ptr));

endproperty

a_illegal_write_ignored: assert property (p_illegal_write_ignored)

else $error("[ASSERT FAIL] wr_ptr changed during illegal write (full)");

cov_illegal_write: cover property (

@(posedge clk) disable iff (!rst_n) (wr_en && full));

//=================================================================

// Property 5: A valid write to an empty FIFO shall make it non-empty

//=================================================================

property p_write_empty_to_nonempty;

@(posedge clk) disable iff (!rst_n)

(wr_en && empty && !full) |=> !empty;

endproperty

a_write_empty_to_nonempty: assert property (p_write_empty_to_nonempty)

else $error("[ASSERT FAIL] FIFO still empty after valid write to empty FIFO");

cov_write_empty: cover property (

@(posedge clk) disable iff (!rst_n) (wr_en && empty && !full));

//=================================================================

// Property 6: A valid read from a full FIFO shall make it non-full

//=================================================================

property p_read_full_to_nonfull;

@(posedge clk) disable iff (!rst_n)

(rd_en && full && !empty) |=> !full;

endproperty

a_read_full_to_nonfull: assert property (p_read_full_to_nonfull)

else $error("[ASSERT FAIL] FIFO still full after valid read from full FIFO");

cov_read_full: cover property (

@(posedge clk) disable iff (!rst_n) (rd_en && full && !empty));

//=================================================================

// Property 7: Data shall be read out in FIFO (FIFO) order

//=================================================================

property p_data_order;

@(posedge clk) disable iff (!rst_n)

expected_valid |-> (dout == expected_data);

endproperty

a_data_order: assert property (p_data_order)

else $error("[ASSERT FAIL] Data ordering violation: expected=%0h got=%0h",

expected_data, dout);

//=================================================================

// Additional coverage to help identify vacuous assertion successes

//=================================================================

cov_full_reached  : cover property (@(posedge clk) disable iff (!rst_n) full);

cov_empty_reached : cover property (@(posedge clk) disable iff (!rst_n) empty);

cov_valid_write   : cover property (@(posedge clk) disable iff (!rst_n) (wr_en && !full));

cov_valid_read    : cover property (@(posedge clk) disable iff (!rst_n) (rd_en && !empty));

endmodule

//=====================================================================

// Bind statement: attaches the checker to every instance of "fifo"

// without modifying the RTL. Internal signals wr_ptr/rd_ptr of the

// fifo module are visible here because bind places the instance

// inside the target module's scope.

//=====================================================================

bind fifo fifo_assertions #(

.DATA_WIDTH(DATA_WIDTH),

.DEPTH(DEPTH),

.PTR_WIDTH(PTR_WIDTH)

) u_fifo_assertions (

.clk    (clk),

.rst_n  (rst_n),

.wr_en  (wr_en),

.rd_en  (rd_en),

.din    (din),

.dout   (dout),

.full   (full),

.empty  (empty),

.wr_ptr (wr_ptr),

.rd_ptr (rd_ptr)

);

//=====================================================================

// Description: Testbench for the synchronous FIFO. Generates clock,

//              reset, and directed stimulus covering all required

//              test cases. Assertions from fifo_assertions.sv are

//              automatically attached via the `bind` statement.

//=====================================================================

module fifo_tb;

localparam int DATA_WIDTH = 8;

localparam int DEPTH      = 8;

logic                  clk;

logic                  rst_n;

logic                  wr_en, rd_en;

logic [DATA_WIDTH-1:0] din;

logic [DATA_WIDTH-1:0] dout;

logic                  full, empty;

int i;

//-----------------------------------------------------------------

// DUT instantiation

//-----------------------------------------------------------------

fifo #(

.DATA_WIDTH(DATA_WIDTH),

.DEPTH     (DEPTH)

) dut (

.clk   (clk),

.rst_n (rst_n),

.wr_en (wr_en),

.rd_en (rd_en),

.din   (din),

.dout  (dout),

.full  (full),

.empty (empty)

);

//-----------------------------------------------------------------

// Clock generation: 10ns period

//-----------------------------------------------------------------

initial clk = 1'b0;

always #5 clk = ~clk;

//-----------------------------------------------------------------

// Reset generation (active-low)

//-----------------------------------------------------------------

initial begin

rst_n = 1'b0;

wr_en = 1'b0;

rd_en = 1'b0;

din   = '0;

repeat (3) @(negedge clk);

rst_n = 1'b1;

end

//-----------------------------------------------------------------

// Tasks for write / read stimulus (single-cycle pulses)

//-----------------------------------------------------------------

task automatic do_write(input [DATA_WIDTH-1:0] data);

@(negedge clk);

wr_en = 1'b1;

din   = data;

@(negedge clk);

wr_en = 1'b0;

endtask

task automatic do_read();

@(negedge clk);

rd_en = 1'b1;

@(negedge clk);

rd_en = 1'b0;

endtask

task automatic do_write_read(input [DATA_WIDTH-1:0] data);

@(negedge clk);

wr_en = 1'b1; din = data;

rd_en = 1'b1;

@(negedge clk);

wr_en = 1'b0; rd_en = 1'b0;

endtask

//-----------------------------------------------------------------

// Simple transaction monitor

//-----------------------------------------------------------------

always @(posedge clk) begin

if (rst_n) begin

if (wr_en) $display("[%0t] WRITE  din=%0h full=%0b empty=%0b",

$time, din, full, empty);

if (rd_en) $display("[%0t] READ   dout=%0h full=%0b empty=%0b",

$time, dout, full, empty);

end

end

//-----------------------------------------------------------------

// Main stimulus sequence (covers all required test cases)

//-----------------------------------------------------------------

initial begin

// Wait for reset release --------------------------------- Test 1

wr_en = 0; rd_en = 0; din = 0;

wait (rst_n == 1'b1);

@(negedge clk);

$display("== Test 1: Reset check (empty expected) ==");

// Test 2: single write ------------------------------------------

$display("== Test 2: Single write ==");

do_write(8'hA1);

// Test 3: single read --------------------------------------------

$display("== Test 3: Single read ==");

do_read();

// Test 4: consecutive writes --------------------------------------

$display("== Test 4: Consecutive writes ==");

do_write(8'h11);

do_write(8'h22);

do_write(8'h33);

// Test 5: consecutive reads ----------------------------------------

$display("== Test 5: Consecutive reads ==");

do_read();

do_read();

do_read();

// Test 6: fill FIFO until full --------------------------------------

$display("== Test 6: Fill FIFO until full ==");

for (i = 0; i < DEPTH; i++) begin

do_write(8'h50 + i[7:0]);

end

// Test 9: attempt write when full (illegal, must be ignored) --------

$display("== Test 9: Attempt write while full ==");

do_write(8'hFF);

// Test 7: empty FIFO until empty asserted ----------------------------

$display("== Test 7: Empty FIFO until empty ==");

for (i = 0; i < DEPTH; i++) begin

do_read();

end

// Test 8: attempt read when empty (illegal, must be ignored) --------

$display("== Test 8: Attempt read while empty ==");

do_read();

// Test 10: FIFO ordering verification --------------------------------

$display("== Test 10: FIFO ordering check ==");

for (i = 0; i < DEPTH; i++) begin

do_write(i[7:0]);

end

for (i = 0; i < DEPTH; i++) begin

do_read();

end

// Extra: simultaneous write/read to exercise boundary transitions

$display("== Extra: simultaneous write & read ==");

do_write(8'hAA);

do_write_read(8'hBB);

do_read();

#50;

$display("Simulation complete at time %0t", $time);

$stop;

end

//-----------------------------------------------------------------

// Waveform dump

//-----------------------------------------------------------------

initial begin

$dumpfile("fifo_tb.vcd");

$dumpvars(0, fifo_tb);

end

endmodule

//=====================================================================

// Description: Synchronous FIFO, 8-bit wide, 8-entry deep, active-low

//              asynchronous reset, single clock domain.

//=====================================================================

module fifo #(

parameter int DATA_WIDTH = 8,

parameter int DEPTH      = 8,

parameter int PTR_WIDTH  = $clog2(DEPTH)

)(

input  logic                   clk,

input  logic                   rst_n,

input  logic                   wr_en,

input  logic                   rd_en,

input  logic [DATA_WIDTH-1:0]  din,

output logic [DATA_WIDTH-1:0]  dout,

output logic                   full,

output logic                   empty

);

// Memory array

logic [DATA_WIDTH-1:0] mem [0:DEPTH-1];

// Pointers have one extra MSB to distinguish full from empty

logic [PTR_WIDTH:0] wr_ptr, rd_ptr;

wire [PTR_WIDTH-1:0] wr_addr = wr_ptr[PTR_WIDTH-1:0];

wire [PTR_WIDTH-1:0] rd_addr = rd_ptr[PTR_WIDTH-1:0];

//-----------------------------------------------------------------

// Status flags

//-----------------------------------------------------------------

assign empty = (wr_ptr == rd_ptr);

assign full  = (wr_ptr[PTR_WIDTH]     != rd_ptr[PTR_WIDTH]) &&

(wr_ptr[PTR_WIDTH-1:0] == rd_ptr[PTR_WIDTH-1:0]);

//-----------------------------------------------------------------

// Write logic

//-----------------------------------------------------------------

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

wr_ptr <= '0;

end else if (wr_en && !full) begin

mem[wr_addr] <= din;

wr_ptr       <= wr_ptr + 1'b1;

end

end

//-----------------------------------------------------------------

// Read logic

//-----------------------------------------------------------------

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

rd_ptr <= '0;

dout   <= '0;

end else if (rd_en && !empty) begin

dout   <= mem[rd_addr];

rd_ptr <= rd_ptr + 1'b1;

end

end

endmodule

WAVEFORM:

TRANSCRIPT:

COMMAND:

vlib work

vmap work work

vlog -sv -mfcu -cuname bind_unit fifo.sv fifo_assertions.sv fifo_bind.sv fifo_tb.sv

vopt +acc=npr work.fifo_tb -o fifo_tb_opt

vsim -assertdebug work.fifo_tb_opt -do wave.do