- RAM Implementation

Source code:

module ram_16x2 (

input        clk,

input        write,

input  [3:0] address,   // 4 bits for 16 locations

input  [1:0] data_in,   // 2 bits (half of 4)

output reg [1:0] data_out

);

// 16 locations, 2 bits each

reg [1:0] mem [0:15];

always @(posedge clk) begin

if (write)

mem[address] <= data_in;

else

data_out <= mem[address];

end

endmodule

TestBench:

module tb_ram_16x2;

reg        clk, write;

reg  [3:0] address;

reg  [1:0] data_in;

wire [1:0] data_out;

ram_16x2 uut (

.clk      (clk),

.write    (write),

.address  (address),

.data_in  (data_in),

.data_out (data_out)

);

initial clk = 0;

always #5 clk = ~clk;

initial begin

$dumpfile("ram.vcd");

$dumpvars(0, tb_ram_16x2);

$display("--------------------------------------");

$display("  ADDR | DATA_IN | OP    | DATA_OUT");

$display("--------------------------------------");

// --- WRITE OPERATIONS ---

write = 1;

address = 4'd0;  data_in = 2'b10; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

address = 4'd1;  data_in = 2'b11; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

address = 4'd2;  data_in = 2'b01; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

address = 4'd3;  data_in = 2'b00; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

address = 4'd4;  data_in = 2'b10; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

address = 4'd5;  data_in = 2'b11; #10;

$display("  %0d    |   %b    | WRITE |   --  ", address, data_in);

// --- READ OPERATIONS ---

write = 0;

$display("--------------------------------------");

address = 4'd0; data_in = 2'bxx; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

address = 4'd5; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

address = 4'd2; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

address = 4'd4; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

address = 4'd3; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

address = 4'd2; #10;

$display("  %0d    |   --   | READ  |   %b  ", address, data_out);

$display("--------------------------------------");

$finish;

end

endmodule

Output:

WaveForm:

- Modulo k counter (k=10)

Source code:

module mod10_counter (

input clk,

input rst,

output reg [3:0] count

);

wire nand_out;

wire reset10;

assign nand_out = ~(count[3] & ~count[2] & count[1] & ~count[0]);

assign reset10 = ~nand_out;

always @(posedge clk or posedge rst or posedge reset10)

begin

if (rst || reset10)

count <= 4'b0000;

else

count <= count + 1;

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg clk;

reg rst;

wire [3:0] count;

mod10_counter uut (

.clk(clk),

.rst(rst),

.count(count)

);

always #5 clk = ~clk;

// Stimulus

initial begin

clk = 0;

rst = 1;

#10 rst = 0;

#150 $finish;

end

initial begin

$dumpfile("wave.vcd");

$dumpvars(0, tb);

end

initial begin

$display("Time\tReset\tCount");

$monitor("%0t\t%b\t%d (%b)",

$time, rst, count, count);

end

endmodule

Output:

WaveForm:

- Rotating letters

Source code:

module left_rotate (

input clk,

input rst,

output reg [7:0] c0,

output reg [7:0] c1,

output reg [7:0] c2,

output reg [7:0] c3,

output reg [1:0] count

);

reg [7:0] temp;

always @(posedge clk or posedge rst)

begin

if (rst)

begin

c0 <= "D";

c1 <= "E";

c2 <= "1";

c3 <= "0";

temp <= 8'd0;

count <= 0;

end

else if (count < 3)

begin

temp = c0;

c0 <= c1;

c1 <= c2;

c2 <= c3;

c3 <= temp;

count <= count + 1;

end

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg clk;

reg rst;

wire [7:0] c0, c1, c2, c3;

wire [1:0] count;

left_rotate uut (

.clk(clk),

.rst(rst),

.c0(c0),

.c1(c1),

.c2(c2),

.c3(c3),

.count(count)

);

always #5 clk = ~clk;

initial begin

clk = 0;

rst = 1;

#10 rst = 0;

#50 $finish;

end

initial begin

$dumpfile("wave.vcd");

$dumpvars(1, tb);

$display("Count   Display");

forever begin

@(posedge clk);

#1;

$display("%0d      %s%s%s%s",

count, c0, c1, c2, c3);

end

end

endmodule

Output:

WaveForm:

- Rotating letters

Source code:

module left_rotate (

input clk,

input rst,

output reg [7:0] c0,

output reg [7:0] c1,

output reg [7:0] c2,

output reg [7:0] c3,

output reg [7:0] c4,

output reg [7:0] c5,

output reg [2:0] count

);

reg [7:0] temp;

always @(posedge clk or posedge rst)

begin

if (rst)

begin

c0 <= "-";

c1 <= "-";

c2 <= "D";

c3 <= "E";

c4 <= "1";

c5 <= "0";

count <= 0;

end

else if (count < 5)

begin

temp = c0;

c0 <= c1;

c1 <= c2;

c2 <= c3;

c3 <= c4;

c4 <= c5;

c5 <= temp;

count <= count + 1;

end

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg clk;

reg rst;

wire [7:0] c0,c1,c2,c3,c4,c5;

wire [2:0] count;

left_rotate uut(

.clk(clk),

.rst(rst),

.c0(c0),

.c1(c1),

.c2(c2),

.c3(c3),

.c4(c4),

.c5(c5),

.count(count)

);

always #5 clk = ~clk;

initial

begin

clk = 0;

rst = 1;

#10 rst = 0;

#100;

$finish;

end

initial

begin

$dumpfile("wave.vcd");

$dumpvars(1,tb);

$display("Count   Display");

forever begin

@(posedge clk);

#1;

$display("%0d      %s%s%s%s%s%s",

count,

c0,c1,c2,c3,c4,c5);

end

end

endmodule

Output:

WaveForm:

- Making a slow counter

Source code:

`timescale 1ns/1ps

module slow_counter(

input clk,

input rst,

output reg [3:0] fast_count,

output reg [3:0] slow_count,

output enable

);

assign enable = (fast_count == 4'd15);

always @(posedge clk or posedge rst)

begin

if (rst)

fast_count <= 4'd0;

else

fast_count <= fast_count + 1'b1;

end

always @(posedge clk or posedge rst)

begin

if (rst)

slow_count <= 4'd0;

else if (enable)

slow_count <= slow_count + 1'b1;

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg clk;

reg rst;

wire [3:0] fast_count;

wire [3:0] slow_count;

wire enable;

slow_counter uut(

.clk(clk),

.rst(rst),

.fast_count(fast_count),

.slow_count(slow_count),

.enable(enable)

);

initial

begin

clk = 0;

forever #5 clk = ~clk;

end

initial

begin

$dumpfile("slow_counter.vcd");

$dumpvars(0, tb);

rst = 1;

#12;

rst = 0;

#350;

$finish;

end

always @(posedge clk)

begin

$display("Time=%0t | Fast=%2d | Enable=%b | Slow=%2d",

$time,

fast_count,

enable,

slow_count);

end

endmodule

Output:

WaveForm:

- Hamming Encoder/Decoder

Bit Position

b7 b6 b5 b4 b3 b2 b1 b0

│ │ │ │ │ │ │ │

d3 p6 d2 p4 d1 p2 d0 p0

Source code:

module hamming_encoder(

input [3:0] data,

output [7:0] code

);

wire p0,p2,p4,p6;

assign p0 = data[0]^data[1]^data[3];

assign p2 = data[0]^data[2]^data[3];

assign p4 = data[1]^data[2]^data[3];

assign p6 = data[0]^data[1]^data[2];

assign code = {

data[3],

p6,

data[2],

p4,

data[1],

p2,

data[0],

p0

};

endmodule

module hamming_decoder(

input [7:0] code,

output reg error

);

wire s0,s2,s4,s6;

assign s0 = code[0]^code[1]^code[3]^code[7];

assign s2 = code[2]^code[1]^code[5]^code[7];

assign s4 = code[4]^code[3]^code[5]^code[7];

assign s6 = code[6]^code[1]^code[3]^code[5];

always @(*)

begin

if({s6,s4,s2,s0}==4'b0000)

error=0;

else

error=1;

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg [3:0] data;

reg [7:0] received;

wire [7:0] code;

wire error;

hamming_encoder ENC(

.data(data),

.code(code)

);

hamming_decoder DEC(

.code(received),

.error(error)

);

initial

begin

$display("Data   Code      Received   Error");

data=4'b1001;

#10;

received=code;

#10;

$display("%b   %b   %b   %b",

data,

code,

received,

error);

received=code;

received[3]=~received[3];

#10;

$display("%b   %b   %b   %b",

data,

code,

received,

error);

$finish;

end

endmodule

Output:

WaveForm:

- Leading zero detector

Source code:

module leading_zero_detector(

input  [7:0] in,

output reg [3:0] count

);

always @(*) begin

casex(in)

8'b1xxxxxxx: count = 0;

8'b01xxxxxx: count = 1;

8'b001xxxxx: count = 2;

8'b0001xxxx: count = 3;

8'b00001xxx: count = 4;

8'b000001xx: count = 5;

8'b0000001x: count = 6;

8'b00000001: count = 7;

8'b00000000: count = 8;

default:     count = 0;

endcase

end

endmodule

TestBench:

`timescale 1ns/1ps

module tb;

reg [7:0] in;

wire [3:0] count;

leading_zero_detector uut(

.in(in),

.count(count)

);

initial begin

$display("Input        Leading Zeros");

in = 8'b00000001; #10;

$display("%b        %d", in, count);

in = 8'b01000000; #10;

$display("%b        %d", in, count);

in = 8'b00011000; #10;

$display("%b        %d", in, count);

in = 8'b00010000; #10;

$display("%b        %d", in, count);

in = 8'b00001000; #10;

$display("%b        %d", in, count);

in = 8'b10011100; #10;

$display("%b        %d", in, count);

in = 8'b00000010; #10;

$display("%b        %d", in, count);

in = 8'b00100001; #10;

$display("%b        %d", in, count);

in = 8'b00000000; #10;

$display("%b        %d", in, count);

$finish;

end

endmodule

Output: