LAB 3

TASK 1

Digital Access Control System:

module digital_access_control(

input  [3:0] secret_code,

input  [3:0] entered_code,

output reg unlock,

output reg warning,

output reg alarm

);

reg [3:0] diff;

integer count;

always @(*) begin

diff = secret_code ^ entered_code;

count = diff[0] + diff[1] + diff[2] + diff[3];

unlock  = 0;

warning = 0;

alarm   = 0;

if(count == 0)

unlock = 1;

else if(count == 1)

warning = 1;

else

alarm = 1;

end

endmodule

TestBench:

//TEST BENCH

`timescale 1ns/1ps

module digital_access_control_tb;

reg [3:0] secret_code;

reg [3:0] entered_code;

wire unlock;

wire warning;

wire alarm;

digital_access_control DUT(

.secret_code(secret_code),

.entered_code(entered_code),

.unlock(unlock),

.warning(warning),

.alarm(alarm)

);

initial begin

$display("-----------------------------------------------------");

$display("Secret   Entered   Unlock   Warning   Alarm");

$display("-----------------------------------------------------");

secret_code = 4'b1010; entered_code = 4'b1010;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

secret_code = 4'b1010; entered_code = 4'b1011;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

secret_code = 4'b1010; entered_code = 4'b1001;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

secret_code = 4'b0101; entered_code = 4'b0101;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

secret_code = 4'b0101; entered_code = 4'b1101;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

secret_code = 4'b0101; entered_code = 4'b1010;

#10;

$display("%b      %b       %b        %b        %b",

secret_code, entered_code,

unlock, warning, alarm);

$display("-----------------------------------------------------");

$finish;

end

endmodule

Output: