Source Code:
`timescale 1ns/1ps

module fifo (

input clk,

input rst,

input wren,

input rden,

input [7:0] din,

output reg [7:0] dout,

output full,

output empty

);

parameter DEPTH = 16;

parameter ADDR_WIDTH = 4;

//(* ram_style = "block" *)

//(* ram_style = "distributed" *)

reg [7:0] mem [0:DEPTH-1];

reg [ADDR_WIDTH-1:0] wr_ptr;

reg [ADDR_WIDTH-1:0] rd_ptr;

reg [4:0] count;

assign full  = (count == DEPTH);

assign empty = (count == 0);

always @(posedge clk or posedge rst)

begin

if(rst)

begin

wr_ptr <= 0;

rd_ptr <= 0;

count  <= 0;

dout   <= 0;

end

else

begin

// Write only

if(wren && !full && !(rden && !empty))

begin

mem[wr_ptr] <= din;

wr_ptr <= wr_ptr + 1;

count <= count + 1;

end

// Read only

else if(rden && !empty && !(wren && !full))

begin

dout <= mem[rd_ptr];

rd_ptr <= rd_ptr + 1;

count <= count - 1;

end

// Read and Write together

else if(wren && !full && rden && !empty)

begin

mem[wr_ptr] <= din;

dout <= mem[rd_ptr];

wr_ptr <= wr_ptr + 1;

rd_ptr <= rd_ptr + 1;

count <= count;

end

end

end

endmodule

Test Bench:
module fifo_tb;

reg clk;

reg rst;

reg wren;

reg rden;

reg [7:0] din;

wire [7:0] dout;

wire full;

wire empty;

fifo uut(

.clk(clk),

.rst(rst),

.wren(wren),

.rden(rden),

.din(din),

.dout(dout),

.full(full),

.empty(empty)

);

// Clock Generation

always #5 clk = ~clk;

// Test Sequence

initial

begin

clk  = 0;

rst  = 1;

wren = 0;

rden = 0;

din  = 0;

#20;

rst = 0;

// WRITE TEST

wren = 1;

din = 8'h11; #10;

din = 8'h22; #10;

din = 8'h33; #10;

din = 8'h44; #10;

din = 8'h55; #10;

wren = 0;

#20;

// READ TEST

rden = 1;

#10;

#10;

#10;

#10;

#10;

rden = 0;

#20;

// OVERFLOW TEST

wren = 1;

repeat(18)

begin

din = din + 1;

#10;

end

wren = 0;

#20;

// UNDERFLOW TEST

rden = 1;

repeat(18)

#10;

rden = 0;

#30;

$finish;

end

initial

begin

$monitor("Time=%0t WR=%b RD=%b DIN=%h DOUT=%h COUNT=%0d FULL=%b EMPTY=%b",

$time,wren,rden,din,dout,uut.count,full,empty);

end

// Waveform Dump

initial

begin

$dumpfile("fifo.vcd");

$dumpvars(0,fifo_tb);

end

endmodule

Output:

Waveform:

ModelSim:

Source Code:
module sequential_multiplier(

input clk,

input reset,

input start,

input [3:0] multiplicand,

input [3:0] multiplier,

output [7:0] product,

output done

);

wire w_LoadM, w_LoadQ, w_ClearA, w_Add, w_ShiftAQ, w_Count;

wire w_q0, w_count_done;

// Instantiate Controller

controller ctrl (

.clk(clk), .reset(reset), .start(start),

.q0(w_q0), .count_done(w_count_done),

.LoadM(w_LoadM), .LoadQ(w_LoadQ), .ClearA(w_ClearA),

.Add(w_Add), .ShiftAQ(w_ShiftAQ), .Count(w_Count),

.Done(done)

);

// Instantiate Datapath

datapath dp (

.clk(clk), .reset(reset),

.multiplicand_in(multiplicand), .multiplier_in(multiplier),

.LoadM(w_LoadM), .LoadQ(w_LoadQ), .ClearA(w_ClearA),

.Add(w_Add), .ShiftAQ(w_ShiftAQ), .Count(w_Count),

.q0(w_q0), .count_done(w_count_done),

.product(product)

);

endmodule

module datapath(

input clk,

input reset,

input [3:0] multiplicand_in,

input [3:0] multiplier_in,

input LoadM,

input LoadQ,

input ClearA,

input Add,

input ShiftAQ,

input Count,

output q0,

output count_done,

output [7:0] product

);

reg [3:0] M;

reg [3:0] A;

reg [3:0] Q;

reg C;

reg [2:0] counter;

// Parallel Adder Logic

wire [4:0] adder_out = A + M;

always @(posedge clk or posedge reset) begin

if (reset) begin

M <= 4'b0;

A <= 4'b0;

Q <= 4'b0;

C <= 1'b0;

counter <= 3'b0;

end else begin

if (LoadM)

M <= multiplicand_in;

if (ClearA) begin

A <= 4'b0;

C <= 1'b0;

end

// Load Multiplier

if (LoadQ)

Q <= multiplier_in;

if (Add) begin

{C, A} <= adder_out;

end

if (ShiftAQ) begin

// Logical right shift: 0 -> C -> A -> Q

{C, A, Q} <= {1'b0, C, A, Q[3:1]};

end

// Counter Logic

if (ClearA)

counter <= 3'b0;

else if (Count)

counter <= counter + 1'b1;

end

end

// Status signals for Controller

assign q0 = Q[0];

assign count_done = (counter == 4);

assign product = {A, Q};

endmodule

module controller(

input clk,

input reset,

input start,

input q0,

input count_done,

// Control signal outputs

output reg LoadM,

output reg LoadQ,

output reg ClearA,

output reg Add,

output reg ShiftAQ,

output reg Count,

output reg Done

);

typedef enum reg [2:0] {

S_IDLE  = 3'b000,

S_START = 3'b001,

S_CHECK = 3'b010,

S_ADD   = 3'b011,

S_SHIFT = 3'b100,

S_DONE  = 3'b101

} state_t;

state_t current_state, next_state;

// State Transition

always @(posedge clk or posedge reset) begin

if (reset) current_state <= S_IDLE;

else current_state <= next_state;

end

always @(*) begin

LoadM = 0; LoadQ = 0; ClearA = 0; Add = 0;

ShiftAQ = 0; Count = 0; Done = 0;

next_state = current_state;

case (current_state)

S_IDLE: begin

if (start) next_state = S_START;

end

S_START: begin

LoadM = 1;

LoadQ = 1;

ClearA = 1;

next_state = S_CHECK;

end

S_CHECK: begin

if (count_done)

next_state = S_DONE;

else if (q0)

next_state = S_ADD;

else

next_state = S_SHIFT;

end

S_ADD: begin

Add = 1;

next_state = S_SHIFT;

end

S_SHIFT: begin

ShiftAQ = 1;

Count = 1;

next_state = S_CHECK;

end

S_DONE: begin

Done = 1;

if (!start) next_state = S_IDLE;

end

endcase

end

endmodule

Test Bench:
module tb_multiplier();

reg clk, reset, start;

reg [3:0] m, q;

wire [7:0] p;

wire done;

sequential_multiplier uut (clk, reset, start, m, q, p, done);

always #5 clk = ~clk;

initial begin

$dumpfile("dump.vcd"); $dumpvars;

clk = 0; reset = 1; start = 0;

m = 4'd10; q = 4'd3; // 10 * 3 = 30 (8'b00011110)

#15 reset = 0;

#10 start = 1;

#10 start = 0;

wait(done);

$display("Result: %d * %d = %d", m, q, p);

#20;

m = 4'd7; q = 4'd5;  // 7 * 5 = 35 (8'b00100011)

start = 1;

#10 start = 0;

wait(done);

$display("Result: %d * %d = %d", m, q, p);

#50 $finish;

end

endmodule

Waveform:

Source Code:

`timescale 1ns/1ps

module lab3_fsm (

input [1:0] SW,      // SW[0] is synchronous reset_n, SW[1] is w

input [0:0] KEY,     // KEY[0] is manual clock

output [9:0] LEDR    // LEDR[8:0] is y8-y0, LEDR[9] is z

);

wire w, reset_n, clk;

assign w = SW[1];

assign reset_n = SW[0];

assign clk = KEY[0];

wire [8:0] y;  // Outputs of flip-flops

wire [8:0] Y;  // Logic feeding flip-flops

// --- Logic Expressions (Derived by Inspection for Modified One-Hot) ---

// According to Step 6: State A is when y0 = 0.

// Therefore, we are in State A if ~y0 is true.

wire state_A = ~y[0];

// Y0 logic: y0 should be 1 if we are in ANY state other than A.

// So if we are in A and get a signal, or if we are already in B-I.

assign Y[0] = 1'b1;

// To State B (y1): If w=0 and (State A OR State F,G,H,I)

assign Y[1] = ~w & (state_A | y[5] | y[6] | y[7] | y[8]);

// To State C (y2): If w=0 and in B

assign Y[2] = ~w & y[1];

// To State D (y3): If w=0 and in C

assign Y[3] = ~w & y[2];

// To State E (y4): If w=0 and in (D or E)

assign Y[4] = ~w & (y[3] | y[4]);

// To State F (y5): If w=1 and (State A OR State B,C,D,E)

assign Y[5] = w & (state_A | y[1] | y[2] | y[3] | y[4]);

// To State G (y6): If w=1 and in F

assign Y[6] = w & y[5];

// To State H (y7): If w=1 and in G

assign Y[7] = w & y[6];

// To State I (y8): If w=1 and in (H or I)

assign Y[8] = w & (y[7] | y[8]);

// --- Instantiate Flip-Flops ---

// Step 2 Requirement: Instantiate nine flip-flops.

// Step 6 Requirement: All flip-flop outputs reset to 0.

genvar i;

generate

for (i = 0; i <= 8; i = i + 1) begin : ffs

d_ff_sync flip_flop (

.d(Y[i]),

.clk(clk),

.reset_n(reset_n),

.q(y[i])

);

end

endgenerate

// --- Output Logic ---

assign LEDR[8:0] = y;

assign LEDR[9] = y[4] | y[8]; // z = 1 if in state E or I

endmodule

// D-Flip-Flop with Synchronous Reset (as per Lab Step 2)

module d_ff_sync (

input d,

input clk,

input reset_n,

output reg q

);

always @(posedge clk) begin

if (!reset_n)

q <= 1'b0;

else

q <= d;

end

endmodule

Test Bench:
`timescale 1ns/1ps

module tb_lab3_fsm();

reg [1:0] SW;

reg [0:0] KEY;

wire [9:0] LEDR;

lab3_fsm uut (.SW(SW), .KEY(KEY), .LEDR(LEDR));

// Manual Clock Task

task tick;

begin

#10 KEY[0] = 1; #10 KEY[0] = 0; #10;

end

endtask

initial begin

$dumpfile("dump.vcd"); $dumpvars;

// Setup initial values

KEY[0] = 0; SW = 2'b00; // Reset active, w=0

// 1. Test Synchronous Reset

tick; // Reset happens on rising edge

$display("State after Reset: %b (Should be all 0s)", LEDR[8:0]);

// 2. Release Reset and apply four 1s

SW[0] = 1; SW[1] = 1; // w = 1

tick; // To State F

tick; // To State G

tick; // To State H

tick; // To State I

$display("State after 1111: %b, z: %b", LEDR[8:0], LEDR[9]);

// 3. Test Overlapping 1

tick; // Stay in State I

$display("State after 11111: %b, z: %b (z should be 1)", LEDR[8:0], LEDR[9]);

// 4. Transition to 0s

SW[1] = 0; // w = 0

tick; // To State B

tick; // To State C

tick; // To State D

tick; // To State E

$display("State after 0000: %b, z: %b", LEDR[8:0], LEDR[9]);

#20 $finish;

end

endmodule

Output:

Waveform:

Source Code:

Test Bench:


Output:

Waveform:

Source Code:
`timescale 1ns / 1ps

module VGA_top (

input  wire clk_50,

input  wire reset_btn,  // Assume active low

output wire vga_hsync, vga_vsync,

output wire [7:0] vga_r, vga_g, vga_b,

output wire [19:0] SRAM_ADDR,

inout  wire [15:0] SRAM_DQ,

output wire SRAM_CE_N, SRAM_OE_N, SRAM_WE_N,

output wire [6:0] hex0, hex1,

output wire led_blink, led_lock

);

wire clk_65, locked, reset_n;

wire [10:0] x, y;

wire video_on;

wire [7:0] color;

// --- PLL / Clock Generation ---

// In EDA Playground, we simulate a 65MHz clock directly.

// On hardware, you would replace this with an Altera/Xilinx PLL IP.

reg clk_65_reg = 0;

reg lock_reg = 0;

always #7.7 clk_65_reg = ~clk_65_reg; // 7.7ns toggle = ~65MHz

initial #100 lock_reg = 1;            // Mock PLL locking

assign clk_65 = clk_65_reg;

assign led_lock = lock_reg;

assign reset_n = reset_btn & lock_reg;

// --- 7-Segment Display (Group 12) ---

// Patterns for common cathode (0 = off, 1 = on)

assign hex1 = 7'b0000110; // "1"

assign hex0 = 7'b1011011; // "2"

// --- Blinking LED (1 second) ---

reg [25:0] blink_cnt;

reg blink_reg;

// ADJUST FOR SIMULATION:

// Hardware needs 32,500,000 for 0.5s toggle.

// Simulation needs a small number to avoid timeout.

parameter BLINK_LIMIT = 26'd1000;

always @(posedge clk_65 or negedge reset_n) begin

if (!reset_n) begin

blink_cnt <= 0;

blink_reg <= 0;

end else begin

if (blink_cnt >= BLINK_LIMIT) begin

blink_cnt <= 0;

blink_reg <= ~blink_reg;

end else begin

blink_cnt <= blink_cnt + 1;

end

end

end

assign led_blink = blink_reg;

// --- VGA Controller (Hierarchy Part 1) ---

vga_controller sync_gen (

.clk_65(clk_65),

.reset_n(reset_n),

.hsync(vga_hsync),

.vsync(vga_vsync),

.video_on(video_on),

.pixel_x(x),

.pixel_y(y)

);

// --- Memory Interface (Hierarchy Part 2) ---

memory_interface mem_ctrl (

.clk_65(clk_65),

.px(x),

.py(y),

.video_on(video_on),

.sram_addr(SRAM_ADDR),

.sram_dq(SRAM_DQ),

.sram_ce_n(SRAM_CE_N),

.sram_oe_n(SRAM_OE_N),

.sram_we_n(SRAM_WE_N),

.pixel_data(color)

);

// Output colors (Grayscale mapping)

assign vga_r = color;

assign vga_g = color;

assign vga_b = color;

endmodule

// --- Sub-Module: VGA Sync Generator ---

module vga_controller (

input wire clk_65, reset_n,

output reg hsync, vsync, video_on,

output wire [10:0] pixel_x, pixel_y

);

// 1024x768 @ 60Hz timing

parameter H_ACTIVE = 1024, H_FP = 24, H_SYNC = 136, H_BP = 160, H_TOTAL = 1344;

parameter V_ACTIVE = 768,  V_FP = 3,  V_SYNC = 6,   V_BP = 29,  V_TOTAL = 806;

reg [10:0] h_cnt, v_cnt;

always @(posedge clk_65 or negedge reset_n) begin

if (!reset_n) begin

h_cnt <= 0; v_cnt <= 0;

end else begin

if (h_cnt == H_TOTAL - 1) begin

h_cnt <= 0;

if (v_cnt == V_TOTAL - 1) v_cnt <= 0;

else v_cnt <= v_cnt + 1;

end else h_cnt <= h_cnt + 1;

end

end

always @(posedge clk_65) begin

hsync <= ~((h_cnt >= (H_ACTIVE + H_FP)) && (h_cnt < (H_ACTIVE + H_FP + H_SYNC)));

vsync <= ~((v_cnt >= (V_ACTIVE + V_FP)) && (v_cnt < (V_ACTIVE + V_FP + V_SYNC)));

video_on <= (h_cnt < H_ACTIVE) && (v_cnt < V_ACTIVE);

end

assign pixel_x = h_cnt;

assign pixel_y = v_cnt;

endmodule

// --- Sub-Module: SRAM Memory Interface ---

module memory_interface (

input wire clk_65,

input wire [10:0] px, py,

input wire video_on,

output wire [19:0] sram_addr,

inout  wire [15:0] sram_dq,

output wire sram_ce_n, sram_oe_n, sram_we_n,

output reg [7:0] pixel_data

);

// Address = Row * 1024 + Col

assign sram_addr = {py[9:0], px[9:0]};

assign sram_ce_n = 1'b0; // Always selected

assign sram_oe_n = 1'b0; // Always output enabled

assign sram_we_n = 1'b1; // Write disabled (Read Only)

always @(posedge clk_65) begin

// If within screen boundaries, read from SRAM data bus

pixel_data <= (video_on) ? sram_dq[7:0] : 8'h00;

end

endmodule

Test Bench:
`timescale 1ns / 1ps

module VGA_top_tb();

reg clk_50;

reg reset_btn;

wire vga_hsync, vga_vsync;

wire [7:0] vga_r, vga_g, vga_b;

wire [19:0] SRAM_ADDR;

wire [15:0] SRAM_DQ;

wire SRAM_CE_N, SRAM_OE_N, SRAM_WE_N;

wire led_blink, led_lock;

// Instantiate UUT

VGA_top uut (

.clk_50(clk_50), .reset_btn(reset_btn),

.vga_hsync(vga_hsync), .vga_vsync(vga_vsync),

.vga_r(vga_r), .vga_g(vga_g), .vga_b(vga_b),

.SRAM_ADDR(SRAM_ADDR), .SRAM_DQ(SRAM_DQ),

.SRAM_CE_N(SRAM_CE_N), .SRAM_OE_N(SRAM_OE_N), .SRAM_WE_N(SRAM_WE_N),

.led_blink(led_blink), .led_lock(led_lock),

.hex0(), .hex1()

);

// Mock SRAM Behavioral Model

// Put the low 8 bits of the address on the data bus so we can see it change

assign SRAM_DQ = (!SRAM_OE_N) ? {8'h00, SRAM_ADDR[7:0]} : 16'hZZZZ;

// Generate 50MHz Clock

initial clk_50 = 0;

always #10 clk_50 = ~clk_50;

initial begin

// For EDA Playground: This creates the wave file

$dumpfile("dump.vcd");

$dumpvars(0, VGA_top_tb);

$display("--- Starting VGA Simulation ---");

reset_btn = 0; // Reset active

#150;

reset_btn = 1; // Release reset

// Wait for Mock PLL to Lock

wait(led_lock == 1);

$display("Time: %t | PLL Locked", $time);

// Run simulation for exactly 2 lines of video to prevent timeout

// 1 line = 1344 clocks. 2 lines approx 2700 clocks.

repeat (2700) @(posedge clk_50);

$display("Time: %t | Simulation Finished Successfully", $time);

$finish;

end

endmodule

Output:

Waveform: