# 1. COMMAND

csh

source ./hpcc-setup-v1.0.csh

echo $UVMHOME

cd /mnt/hgfs/uvm/

run.f

-incdir ../sv

../sv/yapp_pkg.sv

top.sv

2. FILES

yapp_packet.sv

/*-----------------------------------------------------------------

File name : yapp_packet.sv

Description : YAPP packet data item

-------------------------------------------------------------------*/

typedef enum {GOOD_PARITY, BAD_PARITY} parity_type_e;

class yapp_packet extends uvm_sequence_item;

// ---------------------------------------------------------------

// Protocol data

// ---------------------------------------------------------------

rand bit [1:0] addr;

rand bit [5:0] length;

rand bit [7:0] payload[];

bit [7:0] parity;

// ---------------------------------------------------------------

// Control knobs

// ---------------------------------------------------------------

rand parity_type_e parity_type;

rand int packet_delay;

// ---------------------------------------------------------------

// UVM automation

// ---------------------------------------------------------------

`uvm_object_utils_begin(yapp_packet)

`uvm_field_int(addr, UVM_ALL_ON)

`uvm_field_int(length, UVM_ALL_ON)

`uvm_field_array_int(payload,UVM_ALL_ON)

`uvm_field_int(parity, UVM_ALL_ON)

`uvm_field_enum(parity_type_e, parity_type, UVM_ALL_ON)

`uvm_field_int(packet_delay, UVM_ALL_ON)

`uvm_object_utils_end

// ---------------------------------------------------------------

// Constraints

// ---------------------------------------------------------------

// Valid YAPP address

constraint addr_c {

addr inside {[0:3]};

}

// Valid packet length

constraint length_c {

length inside {[1:63]};

}

// Payload size must be equal to packet length

constraint payload_size_c {

payload.size() == length;

}

// Generate good parity more often than bad parity

constraint parity_type_c {

parity_type dist {

GOOD_PARITY := 5,

BAD_PARITY := 1

};

}

// Packet delay between 1 and 20 clock cycles

constraint packet_delay_c {

packet_delay inside {[1:20]};

}

// ---------------------------------------------------------------

// Constructor

// ---------------------------------------------------------------

function new(string name = "yapp_packet");

super.new(name);

endfunction

// ---------------------------------------------------------------

// Calculate parity

// ---------------------------------------------------------------

function bit [7:0] calc_parity();

bit [7:0] parity_value;

parity_value = 8'h00;

// XOR address and length

parity_value = {6'b0, addr} ^ {2'b0, length};

// XOR all payload bytes

foreach (payload[i])

parity_value ^= payload[i];

return parity_value;

endfunction

// ---------------------------------------------------------------

// Set parity

// ---------------------------------------------------------------

function void set_parity();

if (parity_type == GOOD_PARITY)

parity = calc_parity();

else

parity = ~calc_parity();

endfunction

// ---------------------------------------------------------------

// Called automatically after randomization

// ---------------------------------------------------------------

function void post_randomize();

set_parity();

endfunction

endclass : yapp_packet

yapp_pkg.sv

/*-----------------------------------------------------------------

File name : yapp_pkg.sv

Description : YAPP package

-------------------------------------------------------------------*/

package yapp_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

`include "yapp_packet.sv"

endpackage : yapp_pkg

top.sv

/*-----------------------------------------------------------------

File name : top.sv

Description : YAPP packet test

-------------------------------------------------------------------*/

module top;

// ---------------------------------------------------------------

// Import UVM

// ---------------------------------------------------------------

import uvm_pkg::*;

`include "uvm_macros.svh"

// ---------------------------------------------------------------

// Import YAPP package

// ---------------------------------------------------------------

import yapp_pkg::*;

// ---------------------------------------------------------------

// Test

// ---------------------------------------------------------------

initial begin

yapp_packet packet;

// Create packet object

packet = new("packet");

// Generate and print 5 random packets

repeat (5) begin

if (!packet.randomize()) begin

`uvm_error("RANDFAIL", "Packet randomization failed")

end

packet.print();

end

end

endmodule : top