# Lab 3  Creating a Simple UVC

### Complete Lab Manual — Cadence Xcelium + UVM

## 1. Objective

# The objective of this lab is to create the front-end of a UVM Verification Component (UVC) and understand the built-in phases of uvm_component.

# In this lab, we create a YAPP TX UVC consisting of:

# yapp_tx_driver

# yapp_tx_sequencer

# yapp_tx_monitor

# yapp_tx_agent

# yapp_env

# Then integrate the UVC into the testbench and execute the yapp_5_packets sequence.

# 2. Required Directory Structure

# Start from the main UVM directory:

# cd /mnt/hgfs/uvm

# The Lab 3 directory should be:

# lab03_uvc/

# ├── sv/

# │   ├── yapp_packet.sv

# │   ├── yapp_pkg.sv

# │   ├── yapp_tx_seqs.sv

# │   ├── yapp_tx_driver.sv

# │   ├── yapp_tx_sequencer.sv

# │   ├── yapp_tx_monitor.sv

# │   ├── yapp_tx_agent.sv

# │   └── yapp_env.sv

# │

# └── tb/

# ├── router_tb.sv

# ├── router_test_lib.sv

# ├── top.sv

# └── run.f

# Check:

# ls

# Then:

# cd lab03_uvc

# ls

# Expected:

# sv  tb

# 3. Copy Lab 2 Files

# cd /mnt/hgfs/uvm

# cp -R lab02_test/* lab03_uvc/

# Then:

# cd lab03_uvc

# Check:

# ls

# 4. Check UVM Installation

# Check the UVM environment variable:

# echo $UVMHOME

# Expected similar output:

# /usr/Software/CadenceTools/XCELIUM2409/tools/methodology/UVM/CDNS-1.1d

# 5. Create yapp_tx_driver.sv

# Go to the sv directory:

# cd /mnt/hgfs/uvm/lab03_uvc/sv

# Create the driver:

# nano yapp_tx_driver.sv

# Enter:

# class yapp_tx_driver extends uvm_driver #(yapp_packet);

# `uvm_component_utils(yapp_tx_driver)

# function new(string name = "yapp_tx_driver",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual task run_phase(uvm_phase phase);

# forever begin

# seq_item_port.get_next_item(req);

# send_to_dut(req);

# seq_item_port.item_done();

# end

# endtask

# virtual task send_to_dut(yapp_packet packet);

# `uvm_info("YAPP_TX_DRIVER",

# $sformatf("Packet is \n%s", packet.sprint()),

# UVM_LOW)

# #10ns;

# endtask

# endclass

# Save:

# Ctrl + O

# Enter

# Ctrl + X

# Verify:

# cat yapp_tx_driver.sv

# 6. Create yapp_tx_sequencer.sv

# nano yapp_tx_sequencer.sv

# Enter:

# class yapp_tx_sequencer extends uvm_sequencer #(yapp_packet);

# `uvm_component_utils(yapp_tx_sequencer)

# function new(string name = "yapp_tx_sequencer",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# endclass

# Save and check:

# cat yapp_tx_sequencer.sv

# 7. Create yapp_tx_monitor.sv

# nano yapp_tx_monitor.sv

# Enter:

# class yapp_tx_monitor extends uvm_monitor;

# `uvm_component_utils(yapp_tx_monitor)

# function new(string name = "yapp_tx_monitor",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual task run_phase(uvm_phase phase);

# `uvm_info("YAPP_TX_MONITOR",

# "You are in the YAPP TX monitor",

# UVM_LOW)

# endtask

# endclass

# Save:

# 8. Create yapp_tx_agent.sv

# The agent contains:

# agent

# ├── driver

# ├── monitor

# └── sequencer

# Create:

# nano yapp_tx_agent.sv

# Enter:

# class yapp_tx_agent extends uvm_agent;

# `uvm_component_utils_begin(yapp_tx_agent)

# `uvm_field_enum(uvm_active_passive_enum, is_active, UVM_ALL_ON)

# `uvm_component_utils_end

# yapp_tx_monitor   monitor;

# yapp_tx_driver    driver;

# yapp_tx_sequencer sequencer;

# function new(string name = "yapp_tx_agent",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual function void build_phase(uvm_phase phase);

# super.build_phase(phase);

# monitor = yapp_tx_monitor::type_id::create("monitor", this);

# if (is_active == UVM_ACTIVE) begin

# driver = yapp_tx_driver::type_id::create("driver", this);

# sequencer = yapp_tx_sequencer::type_id::create("sequencer", this);

# end

# endfunction

# virtual function void connect_phase(uvm_phase phase);

# if (is_active == UVM_ACTIVE) begin

# driver.seq_item_port.connect(sequencer.seq_item_export);

# end

# endfunction

# endclass

# Save and verify:

# cat yapp_tx_agent.sv

# 9. Create yapp_env.sv

# The environment contains the YAPP TX agent.

# nano yapp_env.sv

# Enter:

# class yapp_env extends uvm_env;

# `uvm_component_utils(yapp_env)

# yapp_tx_agent agent;

# function new(string name = "yapp_env",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual function void build_phase(uvm_phase phase);

# super.build_phase(phase);

# agent = yapp_tx_agent::type_id::create("agent", this);

# endfunction

# endclass

# Save and check:

# cat yapp_env.sv

# 10. yapp_pkg.sv

# First inspect the existing package:

# cat yapp_pkg.sv

# It should contain the package declaration and UVM import.

# Open it:

# nano yapp_pkg.sv

# The include order is important.

# Use:

# package yapp_pkg;

# import uvm_pkg::*;

# `include "uvm_macros.svh"

# `include "yapp_packet.sv"

# `include "yapp_tx_monitor.sv"

# `include "yapp_tx_sequencer.sv"

# `include "yapp_tx_seqs.sv"

# `include "yapp_tx_driver.sv"

# `include "yapp_tx_agent.sv"

# `include "yapp_env.sv"

# endpackage : yapp_pkg

# 11. router_tb.sv

# Go to:

# cd ../tb

# Open:

# nano router_tb.sv

# The testbench should contain a handle for yapp_env.

# Use:

# class router_tb extends uvm_env;

# `uvm_component_utils(router_tb)

# yapp_env yapp;

# function new(string name = "router_tb",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual function void build_phase(uvm_phase phase);

# super.build_phase(phase);

# yapp = yapp_env::type_id::create("yapp", this);

# `uvm_info("ROUTER_TB",

# "Build phase of the testbench is being executed",

# UVM_HIGH)

# endfunction

# endclass

# Save.

# 12. router_test_lib.sv

# Open:

# nano router_test_lib.sv

# Use:

# class base_test extends uvm_test;

# `uvm_component_utils(base_test)

# router_tb tb;

# function new(string name = "base_test",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# virtual function void build_phase(uvm_phase phase);

# super.build_phase(phase);

# uvm_config_wrapper::set(this,

# "tb.yapp.agent.sequencer.run_phase",

# "default_sequence",

# yapp_5_packets::get_type());

# tb = router_tb::type_id::create("tb", this);

# `uvm_info("BASE_TEST",

# "Build phase of the test is being executed",

# UVM_HIGH)

# endfunction

# virtual function void end_of_elaboration_phase(uvm_phase phase);

# uvm_top.print_topology();

# endfunction

# endclass

# class test2 extends base_test;

# `uvm_component_utils(test2)

# function new(string name = "test2",

# uvm_component parent = null);

# super.new(name, parent);

# endfunction

# endclass

# 13. top.sv

# Open:

# nano top.sv

# The Lab 2 random packet generation code should be removed.

# Use:

# module top;

# import uvm_pkg::*;

# `include "uvm_macros.svh"

# import yapp_pkg::*;

# `include "router_tb.sv"

# `include "router_test_lib.sv"

# initial begin

# run_test();

# end

# endmodule : top

# 14. Check All Files

# Go to sv:

# cd ../sv

# Run:

# ls

# Expected:

# yapp_env.sv

# yapp_packet.sv

# yapp_pkg.sv

# yapp_tx_agent.sv

# yapp_tx_driver.sv

# yapp_tx_monitor.sv

# yapp_tx_seqs.sv

# yapp_tx_sequencer.sv

# Then:

# cd ../tb

# Run:

# ls

# Expected:

# clean

# qaLog.txt

# router_tb.sv

# router_test_lib.sv

# run.f

# top.sv

# xcelium.d

# xrun.history

# xrun.log

# 15. Configure run.f

# Open:

# nano run.f

# Use:

# -64

# -uvmhome $UVMHOME

# -incdir ../sv

# ../sv/yapp_pkg.sv

# top.sv

# +UVM_TESTNAME=base_test

# +UVM_VERBOSITY=UVM_HIGH

# Save.

# Check:

# cat run.f

# 16. Compile and Run

# From:

# /mnt/hgfs/uvm/lab03_uvc/tb

# run:

# xrun -f run.f

# 17. Check the UVM Topology

# 18. Answer the Lab Questions

### Question 1

# Does the hierarchy match your expectations?

# Answer:

# Yes. The topology contains the base_test, router_tb,

# yapp_env, yapp_tx_agent, driver, monitor and sequencer

# in the expected UVM hierarchy.

### Question 2

# Sequencer pathname:

# tb.yapp.agent.sequencer

### Question 3

# Value of is_active:

# UVM_ACTIVE

# This means the agent contains both:

# driver

# sequencer

# as well as the monitor.

# 19. Verify yapp_5_packets

# 20. Run the Sequence

Go back:

cd ../tb

Run:

xrun -f run.f

# 21. Verify Exactly 5 Packets

A useful command is:

xrun -f run.f | grep "YAPP_TX_DRIVER"

Or:

xrun -f run.f | grep -c "YAPP_TX_DRIVER"

The second command gives the number of driver packet messages.

# 22. Use a Random Seed

Add this to the end of run.f:

+SVSEED=random

Your final run.f should be:

-64

-uvmhome $UVMHOME

-incdir ../sv

../sv/yapp_pkg.sv

top.sv

+UVM_TESTNAME=base_test

+UVM_VERBOSITY=UVM_HIGH

+SVSEED=random

Run:

xrun -f run.f